`timescale 1ns / 1ps

module risc_core_top (

input  wire        clk,

input  wire        rst_n,

output wire [31:0] IHADDR,

output wire [1:0]  IHTRANS,

output wire        IHWRITE,

output wire [2:0]  IHSIZE,

input  wire [31:0] IHRDATA,

input  wire        IHREADY,

output wire [31:0] DHADDR,

output wire [31:0] DHWDATA,

output wire        DHWRITE,

output wire [2:0]  DHSIZE,

output wire [1:0]  DHTRANS,

input  wire [31:0] DHRDATA,

input  wire        DHREADY,

input  wire        irq_i,

input  wire [2:0]  irq_id_i,

output wire        irq_ack_o,

output wire [31:0] debug_pc,

output wire [31:0] debug_alu_result

);

wire stall_mem;

wire ex_m_unit_busy;

wire hazard_pc_write;

wire hazard_if_id_write;

wire hazard_ctrl_mux;

wire ex_branch;

wire ex_jump;

wire ex_m_ext;

wire stall_front = stall_mem | ex_m_unit_busy | !IHREADY | !hazard_pc_write;

wire [31:0] wb_write_data;

wire [4:0]  wb_rd_addr;

wire        wb_reg_write_en;

wire [31:0] if_pc, if_instr;

wire [31:0] ex_jump_target;

wire [31:0] ex_branch_target;

wire        ex_branch_taken;

wire flush;

assign flush = ex_branch_taken;

fetch_stage IF_INST (

.clk(clk),

.rst_n(rst_n),

.branch_taken(ex_branch_taken),

.branch_target(ex_branch_target),

.pc_write(hazard_pc_write),

.stall_all(stall_front),

.IHADDR(IHADDR),

.IHTRANS(IHTRANS),

.IHWRITE(IHWRITE),

.IHSIZE(IHSIZE),

.IHREADY(IHREADY),

.IHRDATA(IHRDATA),

.pc_out(if_pc),

.instr_out(if_instr)

);

wire [31:0] id_pc, id_instr;

if_id_reg PIPE_IF_ID (

.clk(clk), .rst_n(rst_n),

.flush(flush),

.stall(stall_front | !hazard_if_id_write),

.pc_in(if_pc), .instr_in(if_instr),

.pc_out(id_pc), .instr_out(id_instr)

);

wire [31:0] id_rdata1, id_rdata2, id_imm;

wire [4:0]  id_rs1, id_rs2, id_rd;

wire [2:0]  id_funct3;

wire [6:0]  id_funct7;

wire [1:0]  id_mem_to_reg, id_op1_src;

wire [4:0]  id_alu_ctrl;

wire        id_reg_write, id_mem_read, id_mem_write;

wire        id_alu_src, id_branch, id_jump, id_m_ext;

wire        ex_mem_read;

wire [4:0]  ex_final_rd;

decode_stage ID_INST (

.clk(clk), .rst_n(rst_n),

.instr(id_instr), .pc(id_pc),

.wb_reg_write_en(wb_reg_write_en),

.wb_rd_addr(wb_rd_addr), .wb_data(wb_write_data),

.ex_mem_read(ex_mem_read), .ex_rd_addr(ex_final_rd),

.m_unit_busy(ex_m_unit_busy),

.rdata1(id_rdata1), .rdata2(id_rdata2), .imm(id_imm),

.rs1_addr(id_rs1), .rs2_addr(id_rs2), .rd_addr(id_rd),

.funct3(id_funct3), .funct7(id_funct7),

.reg_write(id_reg_write), .mem_read(id_mem_read),

.mem_write(id_mem_write), .mem_to_reg(id_mem_to_reg),

.alu_src(id_alu_src), .op1_src(id_op1_src),

.branch(id_branch), .jump(id_jump), .m_ext(id_m_ext),

.alu_ctrl(id_alu_ctrl),

.pc_write(hazard_pc_write),

.if_id_write(hazard_if_id_write),

.ctrl_mux_sel(hazard_ctrl_mux)

);

wire [31:0] ex_pc, ex_r1, ex_r2, ex_imm;

wire [4:0]  ex_rs1, ex_rs2, ex_alu_ctrl;

wire [2:0]  ex_f3;

wire [6:0]  ex_f7;

wire [1:0]  ex_op1_src, ex_mem_to_reg;

wire        ex_reg_write, ex_mem_write, ex_alu_src;

id_ex_reg PIPE_ID_EX (

.clk(clk), .rst_n(rst_n),

.flush(flush),

.stall(stall_mem | ex_m_unit_busy | !IHREADY),

.pc_in(id_pc), .pc_out(ex_pc),

.read_data1_in(id_rdata1), .read_data1_out(ex_r1),

.read_data2_in(id_rdata2), .read_data2_out(ex_r2),

.imm_in(id_imm), .imm_out(ex_imm),

.rs1_addr_in(id_rs1), .rs1_addr_out(ex_rs1),

.rs2_addr_in(id_rs2), .rs2_addr_out(ex_rs2),

.rd_addr_in(id_rd), .rd_addr_out(ex_final_rd),

.funct3_in(id_funct3), .funct3_out(ex_f3),

.funct7_in(id_funct7), .funct7_out(ex_f7),

.alu_ctrl_in(id_alu_ctrl), .alu_ctrl_out(ex_alu_ctrl),

.reg_write_in(id_reg_write), .reg_write_out(ex_reg_write),

.mem_read_in(id_mem_read), .mem_read_out(ex_mem_read),

.mem_write_in(id_mem_write), .mem_write_out(ex_mem_write),

.mem_to_reg_in(id_mem_to_reg), .mem_to_reg_out(ex_mem_to_reg),

.alu_src_in(id_alu_src), .alu_src_out(ex_alu_src),

.op_src_in(id_op1_src), .op_src_out(ex_op1_src),

.branch_in(id_branch), .branch_out(ex_branch),

.jump_in(id_jump), .jump_out(ex_jump),

.m_ext_in(id_m_ext), .m_ext_out(ex_m_ext)

);

wire [31:0] ex_alu_res, ex_fwd_rs2;

wire [1:0]  forward_a, forward_b;

wire [31:0] mem_alu_result;

wire        mem_reg_write;

wire [4:0]  mem_rd_addr;

forwarding_unit FWD (

.rs1_ex(ex_rs1), .rs2_ex(ex_rs2),

.reg_write_mem(mem_reg_write), .rd_mem(mem_rd_addr),

.reg_write_wb(wb_reg_write_en), .rd_wb(wb_rd_addr),

.forward_a(forward_a), .forward_b(forward_b)

);

execute_stage EX_INST (

.clk(clk), .rst_n(rst_n),

.ex_pc(ex_pc), .rdata1(ex_r1), .rdata2(ex_r2), .imm(ex_imm),

.funct3(ex_f3), .funct7(ex_f7), .alu_ctrl(ex_alu_ctrl),

.op1_src(ex_op1_src), .alu_src_ctrl(ex_alu_src),

.m_ext_ctrl(ex_m_ext), .jump_ctrl(ex_jump), .branch_ctrl(ex_branch),

.mem_alu_result(mem_alu_result), .wb_data(wb_write_data),

.forward_a(forward_a), .forward_b(forward_b),

.final_result(ex_alu_res),

.branch_target(ex_branch_target),

.branch_taken(ex_branch_taken),

.m_unit_busy(ex_m_unit_busy),

.fwd_rs2_out(ex_fwd_rs2)

);

wire [31:0] mem_write_data, mem_pc4;

wire [2:0]  mem_f3;

wire [1:0]  mem_to_reg_ctrl;

wire        mem_mem_read, mem_mem_write;

ex_mem_reg PIPE_EX_MEM (

.clk(clk), .rst_n(rst_n),

.stall(stall_mem),

.reg_write_in(ex_reg_write),    .reg_write_out(mem_reg_write),

.mem_to_reg_in(ex_mem_to_reg),  .mem_to_reg_out(mem_to_reg_ctrl),

.mem_read_in(ex_mem_read),      .mem_read_out(mem_mem_read),

.mem_write_in(ex_mem_write),    .mem_write_out(mem_mem_write),

.alu_result_in(ex_alu_res),     .alu_result_out(mem_alu_result),

.mem_write_data_in(ex_fwd_rs2), .mem_write_data_out(mem_write_data),

.rd_addr_in(ex_final_rd),       .rd_addr_out(mem_rd_addr),

.funct3_in(ex_f3),              .funct3_out(mem_f3),

.pc_plus4_in(ex_pc + 32'd4),    .pc_plus4_out(mem_pc4)

);

wire [31:0] mem_read_data;

memory_stage MEM_INST (

.clk(clk), .rst_n(rst_n),

.addr(mem_alu_result), .write_data(mem_write_data),

.funct3(mem_f3), .mem_read(mem_mem_read), .mem_write(mem_mem_write),

.DHADDR(DHADDR), .DHWDATA(DHWDATA), .DHWRITE(DHWRITE),

.DHSIZE(DHSIZE), .DHTRANS(DHTRANS),

.DHREADY(DHREADY), .DHRDATA(DHRDATA),

.read_data(mem_read_data), .stall_mem(stall_mem)

);

wire [31:0] wb_mem_data, wb_alu_res, wb_pc4;

wire [1:0]  wb_to_reg_ctrl;

mem_wb_reg PIPE_MEM_WB (

.clk(clk), .rst_n(rst_n),

.stall(stall_mem),

.reg_write_in(mem_reg_write),     .reg_write_out(wb_reg_write_en),

.mem_to_reg_in(mem_to_reg_ctrl),  .mem_to_reg_out(wb_to_reg_ctrl),

.mem_read_data_in(mem_read_data), .mem_read_data_out(wb_mem_data),

.alu_result_in(mem_alu_result),   .alu_result_out(wb_alu_res),

.rd_addr_in(mem_rd_addr),         .rd_addr_out(wb_rd_addr),

.pc_plus4_in(mem_pc4),            .pc_plus4_out(wb_pc4)

);

writeback_stage WB_INST (

.mem_to_reg(wb_to_reg_ctrl),

.alu_result(wb_alu_res),

.mem_data(wb_mem_data),

.pc_plus4(wb_pc4),

.wb_write_data(wb_write_data)

);

// ------------------------------------------------------------------------

// IRQ interface placeholder (until trap/CSR path is integrated)

// ------------------------------------------------------------------------

// Lint-safe unused IRQ handling

wire unused_irq;

assign unused_irq = irq_i | (|irq_id_i);

// Acknowledge if any IRQ comes (temporary behavior)

assign irq_ack_o = irq_i;

assign debug_pc         = id_pc;

assign debug_alu_result = ex_alu_res;

endmodule
`timescale 1ns / 1ps

module fetch_stage (

input  wire        clk,

input  wire        rst_n,

input  wire        branch_taken,

input  wire        jump,

input  wire [31:0] branch_target,

input  wire [31:0] jump_target,

input  wire        stall_all,

output wire [31:0] IHADDR,

output wire [1:0]  IHTRANS,

output wire        IHWRITE,

output wire [2:0]  IHSIZE,

input  wire        IHREADY,

input  wire [31:0] IHRDATA,

output wire [31:0] pc_out,

output wire [31:0] instr_out

);

reg [31:0] pc;

reg [31:0] pc_data_phase;

reg        kill_fetch_resp;

wire advance_pc = IHREADY && !stall_all;

wire redirect   = jump || branch_taken;

wire [31:0] redirect_target = jump ? jump_target : branch_target;

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

pc <= 32'h0001_0000;

end else if (advance_pc) begin

if (redirect)

pc <= redirect_target;

else

pc <= pc + 32'd4;

end

end

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

pc_data_phase   <= 32'h0001_0000;

kill_fetch_resp <= 1'b1;

end else if (advance_pc) begin

pc_data_phase <= pc;

if (redirect)

kill_fetch_resp <= 1'b1;

else

kill_fetch_resp <= 1'b0;

end

end

assign IHADDR  = pc;

assign IHWRITE = 1'b0;

assign IHSIZE  = 3'b010;

assign IHTRANS = (!rst_n || stall_all) ? 2'b00 : 2'b10;

assign pc_out = pc_data_phase;

assign instr_out = (!rst_n || !IHREADY || kill_fetch_resp || (^IHRDATA === 1'bX))

? 32'h0000_0013

: IHRDATA;

Endmodule
`timescale 1ns / 1ps

module decode_stage (

input  wire        clk,

input  wire        rst_n,

input  wire [31:0] instr,

input  wire [31:0] pc,

input  wire        wb_reg_write_en,

input  wire [4:0]  wb_rd_addr,

input  wire [31:0] wb_data,

input  wire        m_unit_busy,

input  wire        ex_mem_read,

input  wire [4:0]  ex_rd_addr,

output wire [31:0] rdata1,

output wire [31:0] rdata2,

output wire [31:0] imm,

output wire [4:0]  rs1_addr,

output wire [4:0]  rs2_addr,

output wire [4:0]  rd_addr,

output wire [2:0]  funct3,

output wire [6:0]  funct7,

output wire        reg_write,

output wire        mem_read,

output wire        mem_write,

output wire        alu_src,

output wire        branch,

output wire        jump,

output wire        m_ext,

output wire [1:0]  mem_to_reg,

output wire [1:0]  op1_src,

output wire [4:0]  alu_ctrl,

output wire        pc_write,

output wire        if_id_write,

output wire        ctrl_mux_sel

);

assign rs1_addr = instr[19:15];

assign rs2_addr = instr[24:20];

assign rd_addr  = instr[11:7];

assign funct3   = instr[14:12];

assign funct7   = instr[31:25];

wire c_mem_read, c_mem_write, c_reg_write, c_alu_src;

wire c_branch, c_jump, c_m_ext;

wire [1:0] c_mem_to_reg, c_alu_op, c_op1_src;

wire [1:0] pc_src_wire;

wire [4:0] c_alu_ctrl;

sign_gen ig (.instr(instr), .imm(imm));

register_f rf (

.clk(clk),

.rst_n(rst_n),

.write_en(wb_reg_write_en),

.rd_addr(wb_rd_addr),

.write_data(wb_data),

.rs1_addr1(rs1_addr),

.rs2_addr2(rs2_addr),

.rs1_data(rdata1),

.rs2_data(rdata2)

);

main_ctrl control_unit (

.opcode(instr[6:0]),

.funct7(funct7),

.mem_read(c_mem_read),

.mem_write(c_mem_write),

.reg_write(c_reg_write),

.mem_to_reg(c_mem_to_reg),

.alu_src(c_alu_src),

.branch(c_branch),

.jump(c_jump),

.m_ext(c_m_ext),

.alu_op(c_alu_op),

.op1_src(c_op1_src),

.pc_src(pc_src_wire)

);

alu_ctrl_32 acu (

.aluop(c_alu_op),

.funct3(funct3),

.funct7(funct7),

.m_ext(c_m_ext),

.alu_ctrl(c_alu_ctrl)

);

hazard_unit hazard_detect (

.id_ex_mem_read(ex_mem_read),

.id_ex_rd(ex_rd_addr),

.rs1_addr(rs1_addr),

.rs2_addr(rs2_addr),

.m_unit_busy(m_unit_busy),

.pc_write(pc_write),

.if_id_write(if_id_write),

.control_mux(ctrl_mux_sel)

);

assign reg_write  = ctrl_mux_sel ? 1'b0 : c_reg_write;

assign mem_write  = ctrl_mux_sel ? 1'b0 : c_mem_write;

assign mem_read   = ctrl_mux_sel ? 1'b0 : c_mem_read;

assign alu_src    = ctrl_mux_sel ? 1'b0 : c_alu_src;

assign branch     = ctrl_mux_sel ? 1'b0 : c_branch;

assign jump       = ctrl_mux_sel ? 1'b0 : c_jump;

assign m_ext      = ctrl_mux_sel ? 1'b0 : c_m_ext;

assign mem_to_reg = ctrl_mux_sel ? 2'b00 : c_mem_to_reg;

assign op1_src    = ctrl_mux_sel ? 2'b00 : c_op1_src;

assign alu_ctrl   = ctrl_mux_sel ? 5'b00000 : c_alu_ctrl;

endmodule
`timescale 1ns / 1ps

module register_f #(parameter width = 32) (

input  wire clk,

input  wire rst_n,

input  wire write_en,

input  wire [4:0] rd_addr,

input  wire [width-1:0] write_data,

input  wire [4:0] rs1_addr1,

input  wire [4:0] rs2_addr2,

output wire [width-1:0] rs1_data,

output wire [width-1:0] rs2_data

);

reg [width-1:0] registers [0:31];

integer i;

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

for (i = 0; i < 32; i = i + 1)

registers[i] <= {width{1'b0}};

end else if (write_en && (rd_addr != 5'd0)) begin

registers[rd_addr] <= write_data;

end

end

assign rs1_data =

(rs1_addr1 == 5'd0) ? {width{1'b0}} :

(write_en && (rd_addr == rs1_addr1) && (rd_addr != 5'd0)) ? write_data :

registers[rs1_addr1];

assign rs2_data =

(rs2_addr2 == 5'd0) ? {width{1'b0}} :

(write_en && (rd_addr == rs2_addr2) && (rd_addr != 5'd0)) ? write_data :

registers[rs2_addr2];

endmodule
`timescale 1ns / 1ps

module main_ctrl(

input  wire [6:0] opcode,

input  wire [6:0] funct7,

output reg       mem_read,

output reg       mem_write,

output reg       reg_write,

output reg [1:0] mem_to_reg,

output reg       alu_src,

output reg       branch,

output reg       jump,

output reg       m_ext,

output reg [1:0] alu_op,

output reg [1:0] op1_src,

output reg [1:0] pc_src

);

always @(*) begin

mem_read   = 1'b0;

mem_write  = 1'b0;

reg_write  = 1'b0;

mem_to_reg = 2'b00;

alu_src    = 1'b0;

branch     = 1'b0;

jump       = 1'b0;

m_ext      = 1'b0;

alu_op     = 2'b00;

op1_src    = 2'b00;

pc_src     = 2'b00;

case (opcode)

// I-TYPE ALU

7'b0010011: begin

reg_write = 1'b1;

alu_src   = 1'b1;

alu_op    = 2'b11;

end

// R-TYPE

7'b0110011: begin

reg_write = 1'b1;

alu_op    = 2'b10;

m_ext     = (funct7 == 7'b0000001);

end

// LOAD

7'b0000011: begin

reg_write  = 1'b1;

mem_read   = 1'b1;

mem_to_reg = 2'b01;

alu_src    = 1'b1;

alu_op     = 2'b00;

end

// STORE

7'b0100011: begin

mem_write = 1'b1;

alu_src   = 1'b1;

alu_op    = 2'b00;

end

// BRANCH

7'b1100011: begin

branch = 1'b1;

alu_op = 2'b01;

pc_src = 2'b01;

end

// JAL

7'b1101111: begin

reg_write  = 1'b1;

jump       = 1'b1;

mem_to_reg = 2'b10;

op1_src    = 2'b01;

pc_src     = 2'b10;

end

// JALR

7'b1100111: begin

reg_write  = 1'b1;

jump       = 1'b1;

alu_src    = 1'b1;

mem_to_reg = 2'b10;

op1_src    = 2'b00;

pc_src     = 2'b11;

end

// LUI: rd = imm

7'b0110111: begin

reg_write  = 1'b1;

alu_src    = 1'b1;

alu_op     = 2'b00;

mem_to_reg = 2'b00;

op1_src    = 2'b10; // zero + imm

end

// AUIPC: rd = pc + imm

7'b0010111: begin

reg_write  = 1'b1;

alu_src    = 1'b1;

alu_op     = 2'b00;

mem_to_reg = 2'b00;

op1_src    = 2'b01; // pc + imm

end

default: begin

// safe NOP

end

endcase

end

endmodule
`timescale 1ns / 1ps

module hazard_unit (

input  wire        id_ex_mem_read,

input  wire [4:0]  id_ex_rd,

input  wire [4:0]  rs1_addr,

input  wire [4:0]  rs2_addr,

input  wire        m_unit_busy,

output wire        pc_write,

output wire        if_id_write,

output wire        control_mux

);

// -------------------------

// LOAD-USE HAZARD

// -------------------------

wire load_use_hazard;

assign load_use_hazard =

id_ex_mem_read &&

(id_ex_rd != 5'd0) &&

((id_ex_rd == rs1_addr) || (id_ex_rd == rs2_addr));

// -------------------------

// M UNIT STALL (FIXED)

// Only stall when EX stage depends on it indirectly

// -------------------------

wire m_stall = m_unit_busy;

// -------------------------

// FINAL STALL

// -------------------------

wire stall = load_use_hazard || m_stall;

// -------------------------

// CONTROL OUTPUTS

// -------------------------

assign pc_write    = ~stall;

assign if_id_write = ~stall;

assign control_mux =  stall;   // insert NOP in ID/EX

endmodule
`timescale 1ns / 1ps

module sign_gen (

input  wire [31:0] instr,

output reg  [31:0] imm

);

wire [6:0] opcode;

assign opcode = instr[6:0];

always @(*) begin

case (opcode)

// =========================

// I-TYPE (LOAD / ALU IMM / JALR)

// =========================

7'b0000011,

7'b0010011,

7'b1100111: begin

imm = {{20{instr[31]}}, instr[31:20]};

end

// =========================

// S-TYPE (STORE)

// =========================

7'b0100011: begin

imm = {{20{instr[31]}},

instr[31:25],

instr[11:7]};

end

// =========================

// B-TYPE (BRANCH)  ✔ FIXED

// imm[12|10:5|4:1|11|0]

// =========================

7'b1100011: begin

imm = {{19{instr[31]}},

instr[31],        // imm[12]

instr[7],         // imm[11]

instr[30:25],     // imm[10:5]

instr[11:8],      // imm[4:1]

1'b0};            // imm[0]

end

// =========================

// U-TYPE (LUI / AUIPC)

// =========================

7'b0110111,

7'b0010111: begin

imm = {instr[31:12], 12'b0};

end

// =========================

// J-TYPE (JAL) ✔ FIXED

// imm[20|10:1|11|19:12|0]

// =========================

7'b1101111: begin

imm = {{12{instr[31]}},   // ✅ FIXED (12 bits)

instr[19:12],

instr[20],

instr[30:21],

1'b0};            // imm[0]

end

// =========================

// DEFAULT

// =========================

default: begin

imm = 32'b0;

end

endcase

end

endmodule
`timescale 1ns / 1ps

module alu_ctrl_32 (

input  wire [1:0] aluop,

input  wire [2:0] funct3,

input  wire [6:0] funct7,

input  wire       m_ext,

output reg  [4:0] alu_ctrl

);

always @(*) begin

// -------------------------

// DEFAULT SAFE STATE

// -------------------------

alu_ctrl = 5'b00000;

case (aluop)

// =========================

// LOAD / STORE / ADDI BASE

// =========================

2'b00: begin

alu_ctrl = 5'b00000; // ADD

end

// =========================

// BRANCH (COMPARE LOGIC)

// =========================

2'b01: begin

// SUB used for comparison

// (zero flag or signed compare in ALU)

alu_ctrl = 5'b00001;

end

// =========================

// R-TYPE / M-EXTENSION

// =========================

2'b10: begin

if (m_ext) begin

case (funct3)

3'b000: alu_ctrl = 5'b10000; // MUL

3'b001: alu_ctrl = 5'b10001; // MULH

3'b010: alu_ctrl = 5'b10010; // MULHSU

3'b011: alu_ctrl = 5'b10011; // MULHU

3'b100: alu_ctrl = 5'b10100; // DIV

3'b101: alu_ctrl = 5'b10101; // DIVU

3'b110: alu_ctrl = 5'b10110; // REM

3'b111: alu_ctrl = 5'b10111; // REMU

default: alu_ctrl = 5'b00000; // SAFE FALLBACK

endcase

end

else begin

case (funct3)

3'b000: alu_ctrl = (funct7[5]) ? 5'b00001 : 5'b00000; // ADD/SUB

3'b111: alu_ctrl = 5'b00010; // AND

3'b110: alu_ctrl = 5'b00011; // OR

3'b100: alu_ctrl = 5'b00100; // XOR

3'b010: alu_ctrl = 5'b01000; // SLT

3'b011: alu_ctrl = 5'b01001; // SLTU

3'b001: alu_ctrl = 5'b01101; // SLL

3'b101: alu_ctrl = (funct7[5]) ? 5'b01111 : 5'b01110; // SRA/SRL

default: alu_ctrl = 5'b00000;

endcase

end

end

// =========================

// I-TYPE ALU

// =========================

2'b11: begin

case (funct3)

3'b000: alu_ctrl = 5'b00000; // ADDI

3'b010: alu_ctrl = 5'b01000; // SLTI

3'b011: alu_ctrl = 5'b01001; // SLTIU

3'b100: alu_ctrl = 5'b00100; // XORI

3'b110: alu_ctrl = 5'b00011; // ORI

3'b111: alu_ctrl = 5'b00010; // ANDI

default: alu_ctrl = 5'b00000;

endcase

end

default: begin

alu_ctrl = 5'b00000;

end

endcase

end

endmodule
`timescale 1ns / 1ps

module id_ex_reg (

input  wire clk,

input  wire rst_n,

input  wire flush,

input  wire stall,

input  wire        reg_write_in,

input  wire [1:0]  mem_to_reg_in,

input  wire        mem_read_in,

input  wire        mem_write_in,

input  wire        alu_src_in,

input  wire [4:0]  alu_ctrl_in,

input  wire        branch_in,

input  wire        jump_in,

input  wire        m_ext_in,

input  wire [1:0]  op_src_in,

input  wire [2:0]  funct3_in,

input  wire [6:0]  funct7_in,

input  wire [31:0] pc_in,

input  wire [31:0] read_data1_in,

input  wire [31:0] read_data2_in,

input  wire [31:0] imm_in,

input  wire [4:0]  rd_addr_in,

input  wire [4:0]  rs1_addr_in,

input  wire [4:0]  rs2_addr_in,

output reg         reg_write_out,

output reg  [1:0]  mem_to_reg_out,

output reg         mem_read_out,

output reg         mem_write_out,

output reg         alu_src_out,

output reg  [4:0]  alu_ctrl_out,

output reg         branch_out,

output reg         jump_out,

output reg         m_ext_out,

output reg  [1:0]  op_src_out,

output reg  [2:0]  funct3_out,

output reg  [6:0]  funct7_out,

output reg  [31:0] pc_out,

output reg  [31:0] read_data1_out,

output reg  [31:0] read_data2_out,

output reg  [31:0] imm_out,

output reg  [4:0]  rd_addr_out,

output reg  [4:0]  rs1_addr_out,

output reg  [4:0]  rs2_addr_out

);

always @(posedge clk or negedge rst_n) begin

// ---------------- RESET ----------------

if (!rst_n) begin

reg_write_out  <= 0;

mem_to_reg_out <= 0;

mem_read_out   <= 0;

mem_write_out  <= 0;

alu_src_out    <= 0;

alu_ctrl_out   <= 0;

branch_out     <= 0;

jump_out       <= 0;

m_ext_out      <= 0;

op_src_out     <= 0;

funct3_out     <= 0;

funct7_out     <= 0;

pc_out         <= 0;

read_data1_out <= 0;

read_data2_out <= 0;

imm_out        <= 0;

rd_addr_out    <= 0;

rs1_addr_out   <= 0;

rs2_addr_out   <= 0;

end

// ---------------- FLUSH (TRUE NOP) ----------------

end else if (flush) begin

reg_write_out  <= 1'b0;

mem_to_reg_out <= 2'b00;

mem_read_out   <= 1'b0;

mem_write_out  <= 1'b0;

alu_src_out    <= 1'b0;

alu_ctrl_out   <= 5'b00000;

branch_out     <= 1'b0;

jump_out       <= 1'b0;

m_ext_out      <= 1'b0;

op_src_out     <= 2'b00;

funct3_out     <= 3'b000;

funct7_out     <= 7'b0000000;

pc_out         <= 32'h0000_0000;

read_data1_out <= 32'h0000_0000;

read_data2_out <= 32'h0000_0000;

imm_out        <= 32'h0000_0000;

rd_addr_out    <= 5'd0;

rs1_addr_out   <= 5'd0;

rs2_addr_out   <= 5'd0;

// ---------------- NORMAL ----------------

else if (!stall) begin

reg_write_out  <= reg_write_in;

mem_to_reg_out <= mem_to_reg_in;

mem_read_out   <= mem_read_in;

mem_write_out  <= mem_write_in;

alu_src_out    <= alu_src_in;

alu_ctrl_out   <= alu_ctrl_in;

branch_out     <= branch_in;

jump_out       <= jump_in;

m_ext_out      <= m_ext_in;

op_src_out     <= op_src_in;

funct3_out     <= funct3_in;

funct7_out     <= funct7_in;

pc_out         <= pc_in;

read_data1_out <= read_data1_in;

read_data2_out <= read_data2_in;

imm_out        <= imm_in;

rd_addr_out    <= rd_addr_in;

rs1_addr_out   <= rs1_addr_in;

rs2_addr_out   <= rs2_addr_in;

end

// else stall → hold previous state

end

endmodule

`timescale 1ns / 1ps

module execute_stage (

input  wire        clk,

input  wire        rst_n,

input  wire [31:0] ex_pc,

input  wire [31:0] rdata1,

input  wire [31:0] rdata2,

input  wire [31:0] imm,

input  wire [2:0]  funct3,

input  wire [6:0]  funct7,

input  wire [4:0]  alu_ctrl,

input  wire [1:0]  op1_src,

input  wire        alu_src_ctrl,

input  wire        m_ext_ctrl,

input  wire        jump_ctrl,

input  wire        branch_ctrl,

input  wire [31:0] mem_alu_result,

input  wire [31:0] wb_data,

input  wire [1:0]  forward_a,

input  wire [1:0]  forward_b,

output wire [31:0] final_result,

output wire [31:0] branch_target,

output wire [31:0] jump_target,

output wire        branch_taken,

output wire        m_unit_busy,

output wire [31:0] fwd_rs2_out

);

// ================= FORWARDING =================

reg [31:0] fwd_rs1, fwd_rs2;

always @(*) begin

case (forward_a)

2'b10: fwd_rs1 = mem_alu_result;

2'b01: fwd_rs1 = wb_data;

default: fwd_rs1 = rdata1;

endcase

case (forward_b)

2'b10: fwd_rs2 = mem_alu_result;

2'b01: fwd_rs2 = wb_data;

default: fwd_rs2 = rdata2;

endcase

end

assign fwd_rs2_out = fwd_rs2;

// ================= ALU INPUT =================

reg [31:0] alu_in_a;

wire [31:0] alu_in_b;

always @(*) begin

case (op1_src)

2'b01: alu_in_a = ex_pc;

2'b10: alu_in_a = 32'h00000000;    // for AUIPC / JAL

default: alu_in_a = fwd_rs1;

endcase

end

assign alu_in_b = (alu_src_ctrl) ? imm : fwd_rs2;

// ================= ALU =================

wire [31:0] std_res;

wire alu_zero;

alu_32 main_alu (

.a(alu_in_a),

.b(alu_in_b),

.alu_ctrl(alu_ctrl),

.res(std_res),

.zero(alu_zero),

.neg(),

.overflow(),

.cout()

);

// ================= M EXT =================

wire [31:0] m_res;

m_extension_unit m_ext_unit (

.clk(clk),

.rst_n(rst_n),

.a(fwd_rs1),

.b(fwd_rs2),

.alu_ctrl(alu_ctrl),

.ex_m_ext(m_ext_ctrl),

.result(m_res),

.busy(m_unit_busy)

);

// ================= FINAL RESULT =================

assign final_result = (m_ext_ctrl) ? m_res : std_res;

// ================= BRANCH LOGIC =================

reg take_br;

always @(*) begin

take_br = 1'b0;

if (branch_ctrl) begin

case (funct3)

3'b000: take_br = alu_zero;     // BEQ

3'b001: take_br = ~alu_zero;    // BNE

default: take_br = 1'b0;

endcase

end

end

// ================= TARGET COMPUTATION (FIXED) =================

wire is_jalr = (jump_ctrl && (funct3 == 3'b000));

wire [31:0] jal_target   = ex_pc + imm;

wire [31:0] branch_tgt   = ex_pc + imm;

wire [31:0] jalr_target  = (fwd_rs1 + imm) & 32'hFFFF_FFFE;

assign branch_target = branch_tgt;

assign jump_target   = is_jalr ? jalr_target : jal_target;

assign branch_taken = (branch_ctrl && take_br);

endmodule
`timescale 1ns / 1ps

module m_extension_unit (

input  wire clk, rst_n,

input  wire [31:0] a, b,

input  wire [4:0]  alu_ctrl,

input  wire ex_m_ext,

output reg  [31:0] result,

output reg         busy

);

wire signed [63:0] mul_ss = $signed(a) * $signed(b);

wire        [63:0] mul_uu = {32'h0, a} * {32'h0, b};

wire signed [63:0] mul_su = $signed({{1'b0}, a}) * $signed({1'b0, b});

wire        a_neg = a[31];

wire        b_neg = b[31];

wire [31:0] a_abs = a_neg ? (32'd0 - a) : a;

wire [31:0] b_abs = b_neg ? (32'd0 - b) : b;

wire [31:0] udiv = (b_abs == 32'd0) ? 32'hFFFFFFFF : (a_abs / b_abs);

wire [31:0] urem = (b_abs == 32'd0) ?a_abs  : (a_abs % b_abs);

wire        quot_neg   = a_neg ^ b_neg;

wire [31:0] div_signed = (b == 32'd0) ? 32'hFFFFFFFF :

quot_neg  ? (32'd0 - udiv) :udiv;

wire [31:0] rem_signed = (b == 32'd0) ?a :

a_neg  ? (32'd0 - urem) :urem;

wire [31:0] div_unsigned = (b == 32'd0) ? 32'hFFFFFFFF : (a / b);

wire [31:0] rem_unsigned = (b == 32'd0) ? a            : (a % b);

always @(*) begin

busy   = 1'b0;

result = 32'b0;

if (ex_m_ext) begin

case (alu_ctrl)

5'b10000: result = mul_ss[31:0];

5'b10001: result = mul_ss[63:32];

5'b10010: result = mul_su[63:32];

5'b10011: result = mul_uu[63:32];

5'b10100: result = div_signed;

5'b10101: result = div_unsigned;

5'b10110: result = rem_signed;

5'b10111: result = rem_unsigned;

default:  result = 32'b0;

endcase

end

end

endmodule
`timescale 1ns / 1ps

module alu_32 (

input  wire [31:0] a,

input  wire [31:0] b,

input  wire [4:0]  alu_ctrl,

output reg  [31:0] res,

output wire        zero,

output wire        neg,

output wire        overflow,

output wire        cout

);

wire is_sub = (alu_ctrl == 5'b00001);

wire [31:0] adder_res;

wire        adder_carry;

wire        adder_overflow;

add_32 #(.w(32)) main_adder_unit (

.a(a),

.b(b),

.sub_in(is_sub),

.res(adder_res),

.carry(adder_carry),

.overflow(adder_overflow)

);

// -------------------------

// ALU MAIN LOGIC

// -------------------------

always @(*) begin

case (alu_ctrl)

5'b00000: res = adder_res;                         // ADD

5'b00001: res = adder_res;                         // SUB

5'b00010: res = a & b;                             // AND

5'b00011: res = a | b;                             // OR

5'b00100: res = a ^ b;                             // XOR

5'b01000: res = ($signed(a) < $signed(b)) ? 1 : 0; // SLT

5'b01001: res = (a < b) ? 1 : 0;                   // SLTU

5'b01101: res = a << b[4:0];                       // SLL

5'b01110: res = a >> b[4:0];                       // SRL

5'b01111: res = $signed(a) >>> b[4:0];             // SRA

default:  res = 32'b0;

endcase

end

// -------------------------

// FLAGS

// -------------------------

assign zero = (res == 32'b0);

assign neg  = res[31];

assign overflow = is_sub ? adder_overflow : 1'b0;

assign cout     = is_sub ? adder_carry    : 1'b0;

endmodule
`timescale 1ns / 1ps

module add_32 #(parameter w = 32)(

input  [w-1 : 0] a, b,

input            sub_in,

output [w-1 : 0] res,

output           carry, overflow

);

wire [w-1 : 0] b_bar = sub_in ? ~b : b;

wire           cin   = sub_in;

// Instantiate the hardware adder

prefix_adder_32 #(.w(w)) adder_inst (

.a(a),

.b(b_bar),

.cin(cin),

.sum(res),

.cout(carry)

);

// Correct Overflow Logic:

// If signs of inputs are same but result sign is different

assign overflow = (a[w-1] == b_bar[w-1]) && (res[w-1] != a[w-1]);

endmodule
`timescale 1ns / 1ps

module prefix_adder_32 #(

parameter w = 32

)(

input  [w-1: 0] a, b,

input           cin,

output [w-1 : 0] sum,

output          cout

);

// This behavioral line guarantees correct carry and sum bits.

// Synthesis tools will map this to optimized CLA/Kogge-Stone hardware.

assign {cout, sum} = a + b + cin;

endmodule
`timescale 1ns / 1ps

module forwarding_unit (

input  wire [4:0] rs1_ex,

input  wire [4:0] rs2_ex,

// MEM stage

input  wire       reg_write_mem,

input  wire [4:0] rd_mem,

// WB stage

input  wire       reg_write_wb,

input  wire [4:0] rd_wb,

output reg  [1:0] forward_a,

output reg  [1:0] forward_b

);

always @(*) begin

// -------------------------

// DEFAULT (NO FORWARDING)

// -------------------------

forward_a = 2'b00;

forward_b = 2'b00;

// -------------------------

// FORWARDING FOR rs1 (A)

// MEM has HIGHER PRIORITY

// -------------------------

if (reg_write_mem && (rd_mem != 5'd0) && (rd_mem == rs1_ex))

forward_a = 2'b10;   // from MEM stage

else if (reg_write_wb && (rd_wb != 5'd0) && (rd_wb == rs1_ex))

forward_a = 2'b01;   // from WB stage

// -------------------------

// FORWARDING FOR rs2 (B)

// MEM has HIGHER PRIORITY

// -------------------------

if (reg_write_mem && (rd_mem != 5'd0) && (rd_mem == rs2_ex))

forward_b = 2'b10;   // from MEM stage

else if (reg_write_wb && (rd_wb != 5'd0) && (rd_wb == rs2_ex))

forward_b = 2'b01;   // from WB stage

end

endmodule
`timescale 1ns / 1ps

module ex_mem_reg (

input  wire clk,

input  wire rst_n,

input  wire stall,

input  wire        reg_write_in,

input  wire [1:0]  mem_to_reg_in,

input  wire        mem_read_in,

input  wire        mem_write_in,

input  wire [31:0] alu_result_in,

input  wire [31:0] mem_write_data_in,

input  wire [4:0]  rd_addr_in,

input  wire [2:0]  funct3_in,

input  wire [31:0] pc_plus4_in,

output reg         reg_write_out,

output reg  [1:0]  mem_to_reg_out,

output reg         mem_read_out,

output reg         mem_write_out,

output reg  [31:0] alu_result_out,

output reg  [31:0] mem_write_data_out,

output reg  [4:0]  rd_addr_out,

output reg  [2:0]  funct3_out,

output reg  [31:0] pc_plus4_out

);

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

reg_write_out      <= 1'b0;

mem_to_reg_out     <= 2'b00;

mem_read_out       <= 1'b0;

mem_write_out      <= 1'b0;

alu_result_out     <= 32'b0;

mem_write_data_out <= 32'b0;

rd_addr_out        <= 5'b0;

funct3_out         <= 3'b000;

pc_plus4_out       <= 32'b0;

end

// ✅ STALL = INSERT BUBBLE (not hold stale control)

else if (stall) begin

reg_write_out      <= 1'b0;

mem_to_reg_out     <= 2'b00;

mem_read_out       <= 1'b0;

mem_write_out      <= 1'b0;

alu_result_out     <= 32'b0;

mem_write_data_out <= 32'b0;

rd_addr_out        <= 5'b0;

funct3_out         <= 3'b000;

pc_plus4_out       <= 32'b0;

end

// normal pipeline flow

else begin

reg_write_out      <= reg_write_in;

mem_to_reg_out     <= mem_to_reg_in;

mem_read_out       <= mem_read_in;

mem_write_out      <= mem_write_in;

alu_result_out     <= alu_result_in;

mem_write_data_out <= mem_write_data_in;

rd_addr_out        <= rd_addr_in;

funct3_out         <= funct3_in;

pc_plus4_out       <= pc_plus4_in;

end

end

endmodule
`timescale 1ns / 1ps

module memory_stage (

input  wire        clk,

input  wire        rst_n,

input  wire [31:0] addr,

input  wire [31:0] write_data,

input  wire [2:0]  funct3,

input  wire        mem_read,

input  wire        mem_write,

output wire [31:0] DHADDR,

output wire [31:0] DHWDATA,

output wire        DHWRITE,

output wire [2:0]  DHSIZE,

output wire [1:0]  DHTRANS,

input  wire        DHREADY,

input  wire [31:0] DHRDATA,

output reg  [31:0] read_data,

output wire        stall_mem

);

assign DHADDR  = addr;

assign DHWDATA = write_data;

assign DHWRITE = mem_write;

assign DHTRANS = (mem_read || mem_write) ? 2'b10 : 2'b00;

assign stall_mem = (mem_read || mem_write) && !DHREADY;

assign DHSIZE =

(funct3 == 3'b000) ? 3'b000 :

(funct3 == 3'b001) ? 3'b001 :

3'b010;

always @(posedge clk or negedge rst_n) begin

if (!rst_n)

read_data <= 32'h0;

else if (mem_read && DHREADY)

read_data <= DHRDATA;

end

endmodule
`timescale 1ns / 1ps

module mem_wb_reg (

input  wire clk,

input  wire rst_n,

input  wire stall,

input  wire reg_write_in,

input  wire [1:0]  mem_to_reg_in,

input  wire [31:0] mem_read_data_in,

input  wire [31:0] alu_result_in,

input  wire [4:0]  rd_addr_in,

input  wire [31:0] pc_plus4_in,

output reg         reg_write_out,

output reg  [1:0]  mem_to_reg_out,

output reg  [31:0] mem_read_data_out,

output reg  [31:0] alu_result_out,

output reg  [4:0]  rd_addr_out,

output reg  [31:0] pc_plus4_out

);

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

reg_write_out     <= 0;

mem_to_reg_out    <= 0;

mem_read_data_out <= 0;

alu_result_out    <= 0;

rd_addr_out       <= 0;

pc_plus4_out      <= 0;

end

// IMPORTANT: HOLD STATE WHEN STALL = 1

else if (!stall) begin

reg_write_out     <= reg_write_in;

mem_to_reg_out    <= mem_to_reg_in;

mem_read_data_out <= mem_read_data_in;

alu_result_out    <= alu_result_in;

rd_addr_out       <= rd_addr_in;

pc_plus4_out      <= pc_plus4_in;

end

end

endmodule
`timescale 1ns / 1ps

module writeback_stage (

input  wire [1:0]  mem_to_reg,

input  wire [31:0] alu_result,

input  wire [31:0] mem_data,

input  wire [31:0] pc_plus4,

output reg  [31:0] wb_write_data

);

always @(*) begin

case (mem_to_reg)

2'b00: wb_write_data = alu_result;

2'b01: wb_write_data = mem_data;

2'b10: wb_write_data = pc_plus4;

default: wb_write_data = 32'h0000_0000;

endcase

end

endmodule